May 1, 2006

Flow map

Reading (Dent chapter 12) is posted in a pdf file on the course website

1. What is a flow map?

- It shows the linear movement between places

- Can be viewed in the two extremes of measurement scale and generalization

- The map is typically generalized due to its inherent complexity

- Interchangeably called (with some slight difference)

  - Desire line: straight line between origin and destination (e.g. commuting flow between traffic analysis zone)

  - Linear cartogram: highly generalized flow map (e.g. subway route map)

2. Three types of flow maps

- Radial flow map: shows flow out of and into the node (e.g. traffic flow map)

- Network flow map: show interconnectivity between places (e.g. airline route map)

- Distributive flow map: show distribution of flow between places (e.g. trade flow map)

3. Designing flow map

- Visual hierarchy can be achieved with visual contrast (e.g. value difference, opaque symbol)

- Map projection has a significant impact on the look of maps such as connectivity and relative location, and thus can be wisely (or poorly) used to organize (or disorganize) the map content

- Line scaling: usually it’s a proportional scale because human perceive width in a linear fashion; alternatively range-grading is adopted when the scaling is not suited to given data range

- Unique solution: because flow map is typically complex, it is not uncommon to have unique solutions such as flow map combined with proportional symbol map